# LOAD BALANCING MEDIPATH SYSTEMS

# Introduction

The following is an introduction to the way load balancing could potentially be implemented with MediPATH systems.  This document in not intended to describe how any particular site is actually deployed.

# Model

The MediPATH Load balancing model consists of 2 main aspects:

- Database Distribution (2 tier distribution)

- TCP load balancing.

This allows for processing load to be separated from database storage and replication and to be spread over multiple servers.

# Database Distribution

D3 database supports distributed architecture through the OSFI (open file systems interface) protocol (Tiger Logic).

Application processing can be separated from data storage by having the user’s log on to any one of a number of separate instances of MediPATH configured as “Processing Servers” to run applications.  These Processing Instances access data stored on one or more “Data Storage instances” of MediPATH.   In 3-tier terms, the Application tier and the middleware tiers are essentially are compressed into the same layer.

Processing servers are typically Virtualised VM Ware Servers that can be duplicated in their entirety to instantly to produce greater capacity.

Data Storage Instances may contain all or a subset of the Critical Data files required for MediPATH.  The key data files or contained wholly in a single instance and exist only in one instance at a time.

# IP Load Balancing

By having multiple instances of MediPATH Processing servers, it is possible to use IP Load balancing to implement load sharing and provide automated redundancy.

Using Red Hat Linux’s IP Load balancing (piranha), users are able be automatically allocated a session on any one of the available identical processing servers.  By adding more server power and/or more  processing servers, very high scalability can be achieved.

In RHES Load Balancing Terms, all of the RHES servers containing MediPATH Instances are considered a Linux Virtual Server (LVS) and appear to the WAN users as a Single Server single IP Address.   The MediPATH Data server and its Hotlogger backup can be configured as the Active and Backup Load Balancing Routers (but do not offer user logons).  The Processing servers are “REAL” servers providing User logon and processing.  Internally, there is a high speed dedicated LANon which all the servers exist.  It is NOT possible due to latency reasons to distribute logon servers over the WAN.

GUI Clients

Company

WAN

LOAD BALANCING D3 SERVER CLUSTER ARCHITCTURE

ODBC

Medipath GUI web service

HTTPS

HTTPS

TELNET

Piranah

OSFI PROTOCOL

Piranah

OSFI PROTOCOL

TELNET

Telnet Clients

Medipath Data Server

D3 Linux V9

RHES 5

(Primary Load balance router)

SVR Cluster (High speed)

LAN

Transactions

Medipath

USER SVR

D3 Linux V9

RHES 5

Medipath

USER SVRS

1..n

D3 Linux V9

RHES 5

Medipath

USER SVR

D3 Linux V9

RHES 5

Medipath

USER SVR

D3 Linux V9

RHES 5

RHES LOAD BALANCED (virtual) USER PROCESSING SERVERS (http://www.redhat.com/software/rha/cluster/piranha/)

OSFI PROTOCOL

Medipath Communications

D3 Linux V9

RHES 5

OSFI PROTOCOL

Medipath Data Hot Logger D3 Linux V9

RHES 5

(Backup Load balance router)

# http://www.redhat.com/software/rha/cluster/piranha/

# IP Load Balancing

IP Load Balancing (often known by its project name, Piranha) provides the ability to load-balance incoming IP network requests across a farm of servers. IP Load Balancing is based on open source Linux Virtual Server (LVS) technology, with significant Red Hat enhancements.

An IP Load Balancing (IPLB) cluster appears as one server, but, in reality, a user from the Web is accessing a group of servers behind a pair of redundant IPLB routers. An IPLB cluster consists of at least two layers. The first layer is composed of a pair of similarly configured Red Hat Enterprise Linux AS or ES systems with Red Hat Cluster Suite installed. One of these nodes acts as the active IPLB router (the other acts as a backup), directing requests from the Internet to the second layer--a pool of servers called real servers. The real servers provide the critical services to the end-user while the LVS router balances the load to these servers.

The active router serves two roles in the cluster:

- Balancing the load on the real servers.

- Checking integrity of the services on each of the real servers.

The figure shows a simple IPLB cluster consisting of two layers. On the first layer are two IPLB routers. The active system directs traffic from the Internet to a variable number of real servers on the second layer, which in turn provide the necessary services.

The simple, two-layered configuration used in the figure is best for clusters serving data that does not change very frequently--such as static Web pages--because the individual real servers do not automatically synchronize data between each node. If high-availability shared data access and update is required, a third layer is added to the topology. Cluster Manager is ideal for this purpose, so both clustering types can be used together to create a high availability three-tier solution with full transactional capabilities.